Aquifer architecture and groundwater exploitation: a regional study of shallow aquifers in the western Niger Delta.

Offeh, Ejeh, Doro and Akpoborie

Abstract

This study presents preliminary results from a regional study that seeks to describe the spatial distribution and development of aquifers, as well as develop a suite of models to describe ground water flow and solute transport in this complex deltaic environment. Such models are essential for groundwater evaluation, water source protection as well as environmental management. In order to build robust three-dimensional aquifer models, a stratigraphic framework of the shallow aquifers exploited in the western Niger Delta needs be established at this initial stage of the project and this is being undertaken from existing geophysical, geospatial and hydrogeological data.  The focus on the shallow horizons is predicated on the fact that a majority of private, municipal and industrial water supply boreholes are terminated at a depth range of 50-300m. Stratigraphic modelling of numerous litho logs from as far west as Ogulagha through Warri/ Sapele to Umunede/Asaba in the northeast and southwards through Ellu, Ughelli to Utorogu/Bomadi has revealed an aquifer architecture of facies heterogeneity that is dominated by sandy lithologies interbedded with thin to relatively thick clay lenses, which may be of significant lateral extent in some areas. However, silty- to clayey-sand horizons occur mostly as transitions between clay and sand facies. Early stratigraphic interpretation reveals a bi-modal vertical facies heterogeneity within the aquifer system, that is evident from a comparison of the vertical distribution of lignitic, clayey and silty facies often interstratified with well-developed aquiferous sands. A prevalence of relatively thin lignite, shale and clay facies revealed in  wells in the north east of the study area and the thicker, more continuous clay facies in the southerly wells, appears to be  absent n wells drilled across the central part of the study area, which show a vertical distribution of well-developed aquiferous sands over a depth range of 50-540ft, with less frequent clay facies truncating sand sections at shallower depths. Spatial development of aquifers follows a trend from north to south of the prograding delta sequence, displaying a seaward thinning of sand facies into thicker continuous clay intervals in the near shallow offshore margins of the Niger Delta as typified by the lithosection revealed by the well drilled in the Forcados area. Regional data collected for the study cover Delta top facies of the Benin formation to the Delta front shallow offshore margins, penetrating depths averaged over 120m.

Figure 1

Wells: Asaba-2, Issele-Uku, Umunede, Ewuru, Ute-Okpu, Idumuje-Unor, Kwale, Abraka, Oben CB, Oben RD, Oben LQ, Amukpe, Sapele-West, Orerokpe, Ozoro, Eriemu-3, Edjovhe, Ughelli-East, Utorogu, RA Ogunu-Warri, Forcados

Figure 2